ADAS

Advanced Driver Assistance System

Camera-Only Modular System for Indian Road Conditions

────────────────────────────────────────────────────────────────────────────────

System Architecture & Technical Workflow

Real-Time  •  Edge-Deployable  •  YOLOv8 + DeepSORT  •  Python 3.10+

# Table of Contents

1. System Overview

2. Technology Stack

3. Module 1: Forward Collision Warning (FCW)

4. Module 2: Lane Assist + Blind Spot Detection (BSD)

5. Module 3: Emergency Vehicle Preemption (EVP)

6. Shared Infrastructure

7. Performance & Optimizations

8. Evaluation Framework

9. How to Run

10. Future Roadmap

11. File Map

# 1. System Overview

A modular Advanced Driver Assistance System (ADAS) for Indian road conditions using camera-only perception, optimized for real-time edge deployment. The system consists of 3 independent modules plus shared infrastructure.

## Modules at a Glance

| Module | Name | Description | Input |
| --- | --- | --- | --- |
| FCW | Forward Collision Warning | Detects vehicles ahead, estimates distance & TTC, warns before collision | Front Camera |
| BSD | Lane Assist + Blind Spot | Detects lane boundaries, identifies blind spot vehicles, advises lane changes | Rear Camera |
| EVP | Emergency Preemption | Detects emergency vehicles via flashing lights + siren audio | Front Camera + Mic |

# 2. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Object Detection | YOLOv8n (Ultralytics) | Real-time vehicle/pedestrian detection (6.5 MB model) |
| Object Tracking | DeepSORT + MobileNet | Multi-object tracking with re-identification |
| Computer Vision | OpenCV 4.8+ | Image processing, BEV transforms, Hough lines, HSV masks |
| Math / Signals | NumPy, SciPy | Polynomial fitting, FFT audio analysis |
| Filtering | FilterPy | Kalman filtering for track smoothing |
| GPU Acceleration | PyTorch CUDA, FP16, TensorRT | Half-precision inference, model optimization |
| System Monitoring | psutil | Memory/CPU profiling for edge deployment |
| Audio (Optional) | PyAudio | Real-time microphone input for siren detection |
| Language | Python 3.10+ | Core language |

# 3. Module 1: Forward Collision Warning (FCW)

Detects vehicles ahead in the ego lane, estimates their distance and closing speed, and warns the driver when a collision is imminent — all using a single front camera.

## Processing Pipeline

Step 1: Frame Input (640×360)

Step 2: YOLOv8n Detection

Step 3: DeepSORT Tracking

Step 4: BEV Lane Detection

Step 5: In-Lane Filtering

Step 6: Distance + TTC Calculation

Step 7: Traffic-Aware Decision

### 3.1 Object Detection (detector.py)

Model: YOLOv8n (6.5 MB) — smallest YOLO variant, optimized for speed

Classes: Pedestrian, Car, Motorcycle, Bus, Truck (COCO classes 0, 2, 3, 5, 7)

FP16: FP16 half-precision on GPU (2× faster inference)

TensorRT: TensorRT engine support (.engine format, 2-3× faster on NVIDIA)

Warmup: First inference pre-warmed to avoid startup latency

Frame Skip: Detection runs every 2nd frame; tracker predicts in between

Confidence Threshold: 0.4

### 3.2 Object Tracking (tracker.py)

Algorithm: DeepSORT with MobileNet appearance embedder

Converts YOLO detections → DeepSORT format [x, y, w, h, conf, class]

Kalman filter for position prediction + cosine distance for re-identification

Maintains TrackState objects with position history (last 10 frames), speed estimates

Centroid-based speed: speed = distance(centroid_t, centroid_{t-1})

Parameters: max_age=10 (delete after 10 lost frames), min_hits=3 (confirm after 3 detections)

### 3.3 Lane Detection (lane_detector.py)

Uses Bird's Eye View (BEV) perspective transform + sliding window — proven approach for lane detection:

1. Perspective Transform: Trapezoidal ROI → rectangular BEV using cv2.getPerspectiveTransform

2. Multi-Channel Thresholding: White lanes (HSV), Yellow lanes (HSV), S-channel (HLS), Sobel edges — combined with morphological cleanup

3. Sliding Window Search: Bottom-half histogram finds lane bases; windows slide upward (40px height, 60px half-width)

4. Polynomial Fitting: 2nd-degree polynomial: x = a·y² + b·y + c for each lane boundary

5. History Smoothing: Average over last 8 fits for stable output

6. Output: Drivable mask + lane polygon + lane info (curvature radius, lane offset, road type)

### 3.4 Collision Decision Engine (collision.py)

Distance Estimation (Pinhole Camera Model):

distance_meters = (real_vehicle_height × focal_length) / bbox_pixel_height

= (1.5m × 500px) / bbox_height

In-Lane Filtering:

Uses cv2.pointPolygonTest to check if vehicle's bottom-center is inside the lane polygon

TTC (Time-to-Collision) Calculation:

closing_speed = (distance_prev − distance_curr) × FPS

TTC = current_distance / closing_speed

Smoothed over 5-frame window using moving average

Traffic-Aware Thresholds (Key Innovation for Indian Roads):

| Traffic Level | Vehicle Count | TTC Critical | TTC Warning | Multiplier |
| --- | --- | --- | --- | --- |
| FREE | < 4 | 2.0s | 4.0s | ×1.0 |
| MODERATE | 4 – 7 | 1.4s | 2.8s | ×0.7 |
| HEAVY | ≥ 8 | 1.0s | 2.0s | ×0.5 |

In heavy Indian traffic, vehicles are always close — reducing thresholds prevents constant false alarms.

### 3.5 Warning Manager (warning_manager.py)

| Level | Name | Trigger |
| --- | --- | --- |
| 0 | Normal | No growth detected |
| 1 | Visual | Growth detected (first frame) |
| 2 | Audio | Sustained growth ≥ 0.5 seconds |
| 3 | Continuous | Rapid growth spike |

Rule: Never jump levels (must escalate 0→1→2→3 sequentially)

Cooldown: 1-second cooldown after Level 2+ alert to prevent spamming

### 3.6 Supporting Modules

ego_path.py — Defines ego vehicle path as center 40% width × bottom 60% height; checks ≥30% horizontal overlap

motion_metrics.py — Bounding box area growth rate + vertical expansion rate with EMA smoothing

traffic_engine.py — Traffic density, speed variance, lane occupancy ratio computation

lane_analyzer.py — Legacy Hough Transform lane detector (Canny edge + HoughLinesP)

# 4. Module 2: Lane Assist + Blind Spot Detection

Detects lane boundaries from a rear camera, identifies vehicles in the left/right blind spots using 3D cube zones, and provides lane change safety advisories.

## 4.1 Blind Spot Analyzer (blind_spot.py)

Bird's Eye View Transform:

Source ROI: [(174,190), (80,359), (496,190), (610,359)] — rear camera perspective

Destination: Full 640×480 rectangle

Inverse transform (Minv) maps results back to camera view

Lane Detection Pipeline:

Step 1: HSV thresholding for white + yellow lane markings

Step 2: Morphological cleanup (close + open, 5×5 kernel)

Step 3: Sliding window polynomial fitting (2nd degree)

Step 4: History smoothing over 10 frames

3D Cube Visualization (Key Innovation):

Generates 2 trapezoidal blind spot zones (left + right) adjacent to detected lanes

Each zone = 140px wide in BEV, from 45% to 100% of frame height

Zones are perspective-transformed back to camera view using cv2.perspectiveTransform

Rendered as 3D wireframe cubes (base + top polygon + connecting vertical edges)

Curvature & Offset:

Real-world conversion: 30m/480px (Y), 3.7m/640px (X, standard lane width)

Radius of curvature from polynomial second derivative

Lane offset = distance of car center from lane center

## 4.2 Lane Change Advisor (lane_change_advisor.py)

Decision Logic:

Check if tracked vehicles' bottom-center falls inside left/right blind spot cubes

Track occupancy persistence (need 3+ consecutive frames to confirm)

Combine with lane offset: drifting > 0.3m toward occupied blind spot → critical warning

| State | Meaning | Action |
| --- | --- | --- |
| SAFE_BOTH | Both sides clear | Lane change safe in either direction |
| SAFE_LEFT / RIGHT | One side clear | Safe to change toward clear side |
| UNSAFE_LEFT / RIGHT | Vehicle in blind spot | Do not change toward occupied side |
| UNSAFE_BOTH | Both blind spots occupied | STAY IN LANE |
| NO_DATA | Insufficient lane data | Cannot determine safety |

Critical Warnings: "DON'T MOVE LEFT!" / "DON'T MOVE RIGHT!" when drifting toward occupied blind spot.

# 5. Module 3: Emergency Vehicle Preemption (EVP)

Detects approaching emergency vehicles using visual (flashing lights) and optionally audio (siren) analysis, then advises the driver to yield.

## 5.1 Visual Light Detector (visual_detector.py)

Emergency Light Color Detection (HSV, tuned for Indian emergency vehicles):

| Color | HSV Low | HSV High | Vehicles |
| --- | --- | --- | --- |
| Red (range 1) | [0, 120, 150] | [10, 255, 255] | Ambulance, Fire |
| Red (range 2) | [165, 120, 150] | [180, 255, 255] | HSV red wrap-around |
| Blue | [100, 120, 150] | [130, 255, 255] | Police |
| Amber/Orange | [10, 100, 200] | [25, 255, 255] | Highway patrol, tow trucks |

Detection Pipeline:

For each tracked vehicle, extract top 30% of bounding box (where lights are)

Apply HSV color masks for red/blue/amber

Filter by contour area: 100px² ≤ area ≤ 5000px²

Track light visibility history (last 30 frames per vehicle)

Blink Frequency Analysis:

Count on↔off transitions in visibility history

Calculate: freq = (transitions/2) / (frames/30fps)

Emergency range: 1.0 – 4.0 Hz

Blinking confirmed → 0.9 confidence; Static light → 0.5 confidence

## 5.2 Siren Detector (siren_detector.py)

FFT-Based Audio Analysis:

Capture audio chunks: 2048 samples @ 22050 Hz via PyAudio

Compute FFT → magnitude spectrum

Extract energy in Indian siren band: 600 – 1600 Hz

Compute siren ratio: siren_energy / total_energy

Track energy history (30-frame window) for temporal analysis

Siren confirmed when: avg energy > 0.05 AND variance > 0.001 (warble pattern)

Falls back gracefully if PyAudio is not installed (visual-only mode).

## 5.3 Preemption Engine (preemption_logic.py)

Multi-Modal Fusion:

fused_score = max(visual_score, audio_score)

if both > 0.3: fused = visual × 0.6 + audio × 0.4 + 0.2  (agreement bonus)

State Machine:

| State | Trigger | Min Frames |
| --- | --- | --- |
| NORMAL | No emergency signals | — |
| ALERT | fused_score ≥ 0.3 | 5 frames |
| YIELD | fused_score ≥ 0.7 | 10 frames |

Never jumps NORMAL → YIELD directly (must pass through ALERT)

Decay: after 15 frames below threshold, counters decrement

Approach Direction: LEFT / RIGHT / BEHIND (based on vehicle position + bbox growth)

# 6. Shared Infrastructure

## 6.1 UI Renderer (ui_renderer.py)

Modern HUD-style overlay with semi-transparent panels. All modules use consistent visual elements:

| Element | Description |
| --- | --- |
| Module Badge | Top-left badge with module name + accent line |
| FPS Counter | Bottom-right, color-coded (green ≥15, amber ≥8, red <8) |
| Status Panels | Dark panels with colored accent bar + key-value metrics |
| Alert Banners | Full-width gradient banner for warnings/dangers |
| Bounding Boxes | Corner-accent style with optional glow effect |
| Lane Overlay | Green semi-transparent drivable area |

Color Scheme: Green (#00E676) = SAFE, Amber (#FFB300) = WARNING, Red (#FF1744) = DANGER, Blue (#29B6F6) = INFO

## 6.2 Module Profiler (profiler.py)

Per-frame: Processing time, output data size (JPEG-compressed), instant FPS

Aggregated: Avg FPS, data rate (KB/s, Mbps), projected storage (MB/hour)

Export: Per-frame metrics to CSV for offline analysis

Edge Readiness: Measures actual bandwidth/storage requirements for real-world deployment

# 7. Performance & Optimizations

| Metric | Value | Notes |
| --- | --- | --- |
| Target FPS | ≥ 15 FPS | Real-time processing |
| Frame Resolution | 640×360 (FCW), 640×480 (BSD, EVP) | Balanced quality/speed |
| YOLO Model Size | 6.5 MB (YOLOv8n) | Smallest YOLO variant |
| Detection Interval | Every 2 frames | Tracker predicts between |
| GPU Acceleration | FP16 + optional TensorRT | 2-3× speedup on NVIDIA |
| Track Memory | 10 frames per object | Rolling history window |
| Latency | < 67ms/frame on GPU | Within real-time budget |

## Optimization Techniques

FP16 Half-Precision: 2× faster inference on NVIDIA GPUs

TensorRT Export: .engine format for 2-3× speedup on edge devices

Frame Skipping: YOLO runs every 2nd frame; tracker predicts in between

ROI Cropping: Crop irrelevant regions before detection (BSD skips top 20%)

Model Warmup: Pre-warm first inference to avoid startup latency spike

History Smoothing: EMA + rolling averages for temporally stable outputs

# 8. Evaluation Framework

Interactive annotation tool (evaluate.py) for measuring detection quality across all modules.

Samples N evenly-spaced frames from video (default: 200)

Runs module pipeline on each frame and displays result

Human annotator classifies each frame: TP / FP / FN / TN

Computes: Precision, Recall, F1 Score, Accuracy

Exports results to JSON + CSV

Supports: lane, lane_fcw, bsd, fcw modules

Commands:

python evaluate.py --module lane --source videos/new.mp4

python evaluate.py --module bsd --source videos/new.mp4

python evaluate.py --module fcw --source videos/new1.mp4

# 9. How to Run

Install dependencies:

pip install -r requirements.txt

Run individual modules:

python -m fcw.main --source videos/new1.mp4

python -m lane_assist.main --source videos/new.mp4

python -m emergency_preemption.main --source videos/video_demo.mp4

python -m emergency_preemption.main --source 0 --microphone

Or use the batch launcher:

run.bat

# 10. Future Roadmap & Improvements

| Area | Current Approach | Proposed Improvement |
| --- | --- | --- |
| Lane Detection | BEV sliding window (classical CV) | Lightweight DNN (Ultra-Fast-Lane) |
| Distance Estimation | Pinhole camera model (approximate) | Monocular depth (MiDaS/ZoeDepth) |
| Multi-Module Fusion | Modules run independently | Unified pipeline, shared inference |
| Siren Detection | FFT energy ratio | Mel-spectrogram + lightweight CNN |
| Night Mode | Fixed HSV thresholds | Adaptive thresholding + IR camera |
| Edge Deployment | Python + PyTorch | ONNX → TensorRT compiled pipeline |
| False Positive Reduction | Traffic-count threshold | ML-based context classifier |
| Vehicle Re-ID | MobileNet embedder (CPU) | GPU-accelerated lightweight CNN |

# 11. Complete File Map

### shared/ — Common Infrastructure

| File | Purpose |
| --- | --- |
| config_common.py | System-wide constants, YOLO/tracker params, UI colors |
| detector.py | YOLOv8n wrapper (FP16, TensorRT, crop optimization) |
| tracker.py | DeepSORT multi-object tracker with TrackState management |
| ui_renderer.py | HUD overlay renderer (badges, panels, alerts, bounding boxes) |
| profiler.py | Performance + bandwidth + storage profiler with CSV export |

### fcw/ — Forward Collision Warning

| File | Purpose |
| --- | --- |
| config.py | FCW thresholds (TTC, traffic, ego path, lane detection) |
| main.py | FCW pipeline entry point and orchestration |
| lane_detector.py | BEV sliding window lane detection (front camera) |
| collision.py | FCW engine (pinhole distance + TTC + traffic-aware decision) |
| ego_path.py | Ego vehicle path definition + overlap checks |
| motion_metrics.py | Bbox growth rate + vertical expansion with EMA smoothing |
| traffic_engine.py | Traffic density / speed / occupancy feature extraction |
| warning_manager.py | 4-level warning state machine with cooldown |
| lane_analyzer.py | Legacy Hough Transform lane detector (kept for reference) |

### lane_assist/ — Lane Assist + Blind Spot Detection

| File | Purpose |
| --- | --- |
| config.py | BSD-specific ROI points, window params, advisory thresholds |
| main.py | Lane Assist pipeline entry point |
| blind_spot.py | BEV lane detection + 3D blind spot cube zone generation |
| lane_change_advisor.py | Lane change safety advisory (fuses BSD + lane offset) |

### emergency_preemption/ — Emergency Vehicle Preemption

| File | Purpose |
| --- | --- |
| config.py | HSV light ranges, siren frequency bands, state thresholds |
| main.py | EVP pipeline entry point |
| visual_detector.py | Emergency light detection (HSV color + blink frequency) |
| siren_detector.py | FFT-based siren detection (600-1600 Hz band) |
| preemption_logic.py | Multi-modal fusion state machine (NORMAL → ALERT → YIELD) |

### Root Files

| File | Purpose |
| --- | --- |
| evaluate.py | Interactive evaluation tool (Precision, Recall, F1, Accuracy) |
| run.bat | Batch launcher for all modules (Windows) |
| requirements.txt | Python dependencies |
| yolov8n.pt | YOLO detection model (6.5 MB) |
| yolov8n-seg.pt | YOLO segmentation model (7 MB, currently unused) |